# Desc: A simply joystick controller

# Author: Andrew Howard

import math

from playerc import *

def main(host, port):

# Connect to the server

client = playerc_client(None, host, port)

if client.connect() != 0:

raise playerc_error_str()

# Open a joystick device

joystick = playerc_joystick(client, 0)

if joystick.subscribe(PLAYERC_READ_MODE) != 0:

raise playerc_error_str()

# Open a position device

position = playerc_position(client, 0)

if position.subscribe(PLAYER_ALL_MODE) != 0:

raise playerc_error_str()

# Enable the motors

position.enable(1)

while 1:

# Read data from the server

pid = client.read()

# Print out current position

if pid == position.info.id:

print '%+.3f %+.3f' % (position.px, position.py)

# Send velocity commands to the robot

if pid == client.id:

vel_x = -joystick.py * 0.50

vel_yaw = -joystick.px * math.pi / 4

position.set_cmd_vel(vel_x, 0, vel_yaw, 1)

return

if __name__ == '__main__':

main('localhost', 6665)